program prepcell

C

Implicit None

C

CCCCCCCCCCCCCCCCCCCCCCCCCCCCCCCCCCCCCCCCCCCCCC

C

C Program preliminaries before compilation:

C

C	(1) remove either the C SP2 or C SU or C GF comment headers

C	(2) save this document as a text file before compilation

C

C

CCCCCCCCCCCCCCCCCCCCCCCCCCCCCCCCCC

C This program fills the microgravity shear cell with

C an array of spheres.  © Michel Louge May 1, 1996.

C

C Modified for variable bumpiness of the outer boundary

C		MYL 5/3/96.

C

C Modified to allow hexagonal or square packings of the

C bumpy boundary.  MYL 5/14/96.

C

C Modified to allow variable packings of the outer boundary.

C MYL 5/16/96.

C

C Modified to allow boundaries with cylindrical bumps.

C MYL 5/20/96.

C

C Modified to limit variable packings to a subsection of each

C region.  MYL 6/3/96.

C

C Modified to permit different particle sizes.  MYL 7/1/96.

C

C Modified for compatibility with new escape reinsertion

C procedure.  MYL 7/12/96.  MYL 7/15/96.  New I/O files MYL 7/17/96.

C

C Modified to permit insertion of additional "purgatory" spheres

C using the McGSCv15.f-7/17/96 code. MYL 7/18/96.

C

C Modified to permit simulation of an inner chain.  MYL 7/25/96

C Modified to permit depth profiles in FD flow.  MYL 8/1/96.

C Modified to output the minimum value of NNesc.  MYL 8/16/96.

C

C Modified to allow filling of the FD region only and hence to reach

C greater values of the solid volume fraction there. MYL 7/30/97.

C

C Compatible with McGSCv36.f-3/28/98.

C				ATTENTION:

C	All EXP files generated before this version must be updated

C	by replacing the single line of impact properties in the previous

C	EXP files by five lines in this version; the first line contains

C	properties for binary impacts, as well as impacts with the

C	inner and outer bumps; the second line contains properties for

C	impacts with side walls.  MYL 3/28/98.

C

C Latest modification to v36: make it compatible with the SP2.  MYL 5/7/98

C

C Modified to run on GrainFlow (replace all random number calls with

C RAN1.F from Numerical Recipes.  MYL 3/2/99.

C

C Begin changes of version 39e.

C

C	Compatible with version 39e.	MYL 11/19/99.

C

C End changes of version 39e.

C

C Begin changes of version 39f.

C

C Compatible with McGSCv39f.f-11/27/99.  Permits simulations of

C a quarter circular section of the cell.  MYL 11/27/99.

C

C

C End changes of version 39f.

C

C

C Begin changes of version 39h.

C

C Compatible with McGSCv39h.f-03/05/00.  MYL 3/6/00.

C

C End changes of version 39h.

C

C Variables:

C NN		maximum number of spheres

C fname	output file name

C flag		main loop control

C

C xlen		length of the straight section

C ylen		height of the straight section

C zlen		depth of the cell

C outrad	outer radius of the circular section

C inrad	inner radius of the curved section

C

C ran0	random number routine from numerical recipes

C	NOTE:	all variables in ran0 are either integer*2 or real*4

C			in initial, make randum real*8 as a buffer to avoid

C			segmentation faults. (Not used with the SP2).

C randum	buffer variable for ran0

C

C	Note:	On the SP2, all Ran0 statements have been replaced by

C		calls to the subroutine "CALL RANDOM_SEED" for

C		initialization and "Call RANDOM_NUMBER(HARVEST)"

C		for subsequent random number generation.

C		(See http://www.nsc.liu.se/~boein/f77to90/a5.html#section21c )

C		To create the version suitable for the Sparc station,

C		copy all statements marked with C  SU red comment headers,

C		remove C  SU red headers to activate the lines and do not

C		activate any statement with C  SP2 red comment headers.

C		Conversely,

C		to create the version suitable for the SP2, copy all statements

C		with the C  SP2 red comment headers, remove C  SP2 and do not

C		activate any statement with C  SU comment headers.

C		For GrainFlow, remove the C GF comment header instead.

C

C

C std		granular agitation

C

C nxlen	integer number of grid cells in xlen

C nylen	integer number of grid cells in ylen

C nzlen	integer number of grid cells in zlen

C ninrad	integer number of grid cells in inrad

C

C cell		grid cell width

C OK		=1, OK to continue

C

C Rcriti	inner critical radius of curved regions

C Rcrito	outer critical radius of curved regions

C

C dist		distance from sphere center to roller axis

C test1,test2	test variables

C xc1,yc1	coordinates of the axis of the left roller

C xc2,yc2	coordinates of the axis of the right roller

C

C dbel		diameter of belt spheres

C dwal	diameter of wall spheres

C waspx	spacing of wall spheres along the wall

C waspz	spacing of wall spheres across the wall

C waspx1	spacing of wall spheres along the wall in region 1

C waspz1	spacing of wall spheres across the wall in region 1

C waspx2	ditto in region 2

C waspz2

C waspx3	ditto in region 3

C waspz3

C waspx4	ditto in region 4

C waspz4

C bespx	spacing of belt spheres along the belt

C bespz	spacing of belt spheres across the belt

C walen	length of the wall

C belen	length of the belt

C

C flagc1	T =>	region 1 of outer wall has spherical bumps

C		F =>				cylindrical bumps

C flagc2	ditto region 2

C flagc3	ditto region 3

C flagc4	ditto region 4

C flagcb	ditto inner boundary

C

C waspx10	initial spacing of wall spheres along the wall in region 1

C waspx1f	final spacing of wall spheres along the wall in region 1

C waspx20	ibid region 2

C waspx2f	ibid region 2

C waspx30	ibid region 3

C waspx3f	ibid region 3

C waspx40	ibid region 4

C waspx4f	ibid region 4

C

C fact		rescaling factor of hemisphere spacing

C

C Nwax	number of spheres along the wall

C Nwaz	number of spheres across the wall

C Nwax1	number of spheres along the wall in region 1

C Nwaz1	number of spheres across the wall in region 1

C Nwax2	ditto in region 2

C Nwaz2

C Nwax3	ditto in region 3

C Nwaz3

C Nwax4	ditto in region 4

C Nwaz4

C Nbex	number of spheres along the belt

C Nbez	number of spheres across the belt

C

C frac1	for variable spacing, fraction of region 1 with variable spacing

C frac2	ditto region 2

C frac3	ditto region 3

C frac4	ditto region 4

C

C Nwaxf1	# of spheres along x in fixed part of variably spaced region 1

C Nwaxf2	ditto region 2

C Nwaxf3	ditto region 3

C Nwaxf4	ditto region 4

C

C waspxf1	fixed spacing in part of region 1 that has fixed spacing

C waspxf2	ditto region 2

C waspxf3	ditto region 3

C waspxf4	ditto region 4

C

C con		sphere volume fraction in VOLUME

C condes	desired volume fraction

C volume	volume in the region bounded by the

C		crests of the boundary spheres

C Ndesired	number of spheres corresponding to CON

C		and VOLUME

C dil		estimated dilatation of the hexagonal packing

C safety	safety factor on the dilatation

C

C Nx		number of spheres in a row

C Nz		number of rows in a layer

C Ny		number of layers

C Np		total number of interior particles

C

C xmax	value of x beyond which no center can exist

C ymax	value of y beyond which no center can exist

C zmax	value of z beyond which no center can exist

C

C xp,yp,zp	intermediate sphere center location

C dx,dy,dz	sphere spacing in three directions for hexagonal pack

C

C x,y,z	sphere center location

C u,v,w	sphere velocity components

C

C vel		magnitude of the velocity

C vbelt	inner wall "belt" velocity

C vwall	outer wall velocity

C factor	rescale factor of wall and belt velocities

C Up		x-velocity component of interior sphere

C Vp		y-velocity component of interior sphere

C theta	angle between vertical and unit vector

C		from roller axis to sphere center

C

C i		integer index for loops

C

C nul		zero

C time0	initial time (zero)

C dt		time increment

C

C				New in v36:

C ed1,mu1,beta01		e,µ,β0 for species 1 spheres

C ed2,mu2,beta02		ditto for species 2

C ed12,mu12,beta012	ditto for species 1 impacting species 2

C eib1,muib1,beta0ib1	e,µ,β0 for species 1 on inner bumps or flats

C eib2,muib2,beta0ib2	ditto species 2

C eob1,muob1,beta0ob1	e,µ,β0 for species 1 on outer bumps or flats

C eob2,muob2,beta0ob2	ditto species 2

C enw1,munw1,beta0nw1	e,µ,β0 for species 1 on near wall

C enw2,munw2,beta0nw2	ditto species 2

C efw1,mufw1,beta0fw1	e,µ,β0 for species 1 on far wall

C efw2,mufw2,beta0fw2	ditto species 2

C

C gravx	gravity in the x-direction

C gravy	gravity in the y-direction

C gravz	gravity in the z-direction

C

C nxa		x-index of first grid column within shear cell

C nxb		x-index just past left belt roller radius (inside roller)

C nxc		last x-index in left curved section

C nxd		first x-index in right curved section

C nxe		x-index just before right belt roller radius (inside roller)

C nxf		x-index of last grid column within shear cell

C

C nya		y-index of bottom grid row within shear cell's bottom section

C nyb		y-index of last grid row within bottom straight section

C nyc		y-index of first grid row within top straight section

C nyd		y-index of top grid row within shear cell's top section

C

C nza		z-index of first grid layer

C nzb		z-index of last grid layer

C

C Ngy	# of statistics grid cells across YLEN

C Ngx	# of statistics grid cells across XLEN

C Ngz		# of statistics grid cells across ZLEN

C Nalpha	# of statistics grid cells along curved section

C

C cellx	statistics grid cell length

C celly	statistics grid cell height

C cellz	statistics grid cell depth (for FD flow)

C errorx	relative change between cellx and celly

C errorz	relative change between cellz and celly

C errora	relative change between area of straight

C		column and fan

C areacell	area of elementary column in straight section

C areafan	area of elementary fan in curved section

C

C flagw	True => uniform bumpiness of fixed outer wall

C

C flagb	True => hexagonal packing of uniform outer wall

C flagb1	True => hexagonal packing of outer wall region 1

C flagb2	True => hexagonal packing of outer wall region 2

C flagb3	True => hexagonal packing of outer wall region 3

C flagb4	True => hexagonal packing of outer wall region 4

C flagbb	True => hexagonal packing of inner "belt" wall

C

C text..	texture; 1=hexagonal; 0 = square packing

C

C text1	texture of the region 1 of the outer wall

C text2	texture of the region 2 of the outer wall

C text3	texture of the region 3 of the outer wall

C text4	texture of the region 4 of the outer wall

C textb	texture of the inner wall "belt"

C

C flagw1	True => variable packing of outer wall region 1

C flagw2	ibid region 2

C flagw3	ibid region 3

C flagw4	ibid region 4

C

C Nw1	number of hemispheres in outer wall region 1

C Nw2	number of hemispheres in outer wall region 2

C Nw3	number of hemispheres in outer wall region 3

C Nw4	number of hemispheres in outer wall region 4

C Nb		number of hemispheres in inner wall

C

C dint1	diameter of species 1

C dint2	ditto species 2

C fracpsd1	number fraction of species 1 in the mixture

C dmax	maximum sphere diameter (interior and wall)

C dmin	minimum sphere diameter (interior and wall)

C imax	maximum number of cells to be searched on each

C		side of the grid cell of the home sphere

C

C flagspec(NN)	logical; T => sphere is of species 1 (default)

C Npp1		desired number of spheres of species 1

C Np1		present number of spheres of species 1

C radius	working variable

C

C dens2	density of the second species (species 1 density is always one)

C

C npurg	number of spheres in the purgatory (default = 0)

C countwait	number of spheres awaiting reinsertion (default = 0)

C

C imaxwall	max excursion away from first grid cell adjacent to wall

C		for interaction with the wall

C

C gerinul	zero element of array geri used in main program

C

C nx0		initial grid1 x-index of reinsertion sub-region

C nxfin	last grid1 x-index of reinsertion sub-region

C ny0,nyfin	ditto y-index

C nz0,nzfin	ditto z-index

C nxdomain	integer length of the reinsertion sub-region in x-direction

C nydomain	ditto y-direction

C nzdomain	ditto z-direction

C nreinx	number of reinsertion points in region 1 or 3 along x-direction

C nreiny	ditto y-direction

C nreinz	ditto z-direction

C nrein	total number of reinsertion points = 2*nreinx*nreiny*nreinz

C

C deltaN1	additional number of species 1 spheres to place in FIFO

C deltaN	additional number of FIFO spheres of both species

C addfrac	fraction of species 1 in the additional set of FIFO spheres

C addNp1	additional number of species 1 spheres

C Ntotal	total number of spheres

C		(interior+FIFO+wall+belt+4 flat walls)

C flagerror	logical		T=> respecify xlen, ylen or zlen

C

C chspx	length of a chain link in x-direction (assumed flexible)

C lispx	spacing of bumps on chain link in x-direction

C hecha	height of center of bumps above the chordal line joining two

C		successive chain pins

C Nlix	number of bump rows on link along x-direction

C Nchax	total number of chain links

C flagchain	logical		T => chain geometry

C				F => continuous belt geometry

C chalen	length of the chain

C charad	radius of the chain at the pins

C

C FD		=1	flow is FD

C		≠1	flow is not FD

C flagFD	logical	T =>	flow is FD

C			F =>	flow is not FD

C nx1		first grid column in Grid1 where FD flow is sought

C nx2		last grid column in Grid1 where FD flow is sought

C x1FD	x-coordinate of the left face of the FD test region

C x2FD	ditto the right face

C

C samepar	working flag to find out if species 2 and species 1 share the

C		same impact parameters.

C

C Origin of coordinate system is located at the lower left corner

C of the shear cell, at the intersection of the lower wall, the vertical

C plane tangent to the left roller and the side lid of the cell.

C

CCCCCCCCCCCCCCCCCCCCCCCCCCCCCCCCCC

C

Integer*4 NN

Parameter (NN=150000)

C

character*12 fname

real*8 pi

real*8 xlen,ylen,zlen,outrad,inrad

real*8 cell

real*8 xc1,yc1,xc2,yc2

real*8 dbel,dwal

real*8 waspx,waspz

real*8 waspx1,waspz1,waspx2,waspz2

real*8 waspx3,waspz3,waspx4,waspz4

real*8 bespx,bespz

real*8 walen,belen

integer*4 Nwax,Nwaz,Nbex,Nbez

integer*4 Nwax1,Nwaz1,Nwax2,Nwaz2

integer*4 Nwax3,Nwaz3,Nwax4,Nwaz4

integer*4 Np,Nx,Ny,Nz

real*8 con,volume,dil,safety

integer*4 Ndesired

real*8 xp,yp,zp,dx,dy,dz

real*8 Rcriti,Rcrito,dist,test1,test2

real*8 xmax,ymax,zmax

integer*4 OK

integer*4 i

real*8 vel,vbelt,vwall,Up,Vp,theta,std,factor,dt

integer*4 nxlen,nylen,nzlen,ninrad

integer*4 nxa,nxb,nxc,nxd,nxe,nxf

integer*4 nya,nyb,nyc,nyd

integer*4 nza,nzb

integer*4 Ngx,Ngy,Nalpha,Ngz

real*8 cellx,celly,errorx,errora,areacell,areafan

real*8 cellz,errorz

C

Real*8 x(NN),y(NN),z(NN),u(NN),v(NN),w(NN)

C

real*8 nul,time0

C	real*8 ed,ew,mu,muw,beta0,beta0w

C

C New parameters in v36:

C

real*8				ed1,mu1,beta01,

&					ed12,mu12,beta012,

&					ed2,mu2,beta02,

&					eib1,muib1,beta0ib1,

&					eib2,muib2,beta0ib2,

&					eob1,muob1,beta0ob1,

&					eob2,muob2,beta0ob2,

&					enw1,munw1,beta0nw1,

&					enw2,munw2,beta0nw2,

&					efw1,mufw1,beta0fw1,

&					efw2,mufw2,beta0fw2

integer*4 samepar

C

real*8 gravx,gravy,gravz

C

logical flag,flagw,flagb,flagb1,flagb2,flagb3,flagb4,flagbb

logical flagc1,flagc2,flagc3,flagc4,flagcb

integer*4 text1,text2,text3,text4,textb

integer*4 Nw1,Nw2,Nw3,Nw4,Nb

C

logical flagw1,flagw2,flagw3,flagw4

real*8 waspx10,waspx1f,waspx20,waspx2f

real*8 waspx30,waspx3f,waspx40,waspx4f

real*8 fact

real*8 frac1,frac2,frac3,frac4

integer*4 Nwaxf1,Nwaxf2,Nwaxf3,Nwaxf4

real*8 waspxf1,waspxf2,waspxf3,waspxf4

C

integer*4 imax

integer*4 Npp1,Np1

real*8 fracpsd1,dmin,dmax,dint1,dint2,radius

C SU	real*4 ran0

C GF	real*8 ran1

real*8 randum

real*8 dens2

C

logical flagspec(NN)

C

integer*4 nreinx,nreiny,nreinz

integer*4 npurg,nrein,countwait

integer*4 imaxwall,gerinul

integer*4 nx0,nxfin,ny0,nyfin,nz0,nzfin

integer*4 nxdomain,nydomain,nzdomain

C

real*8 condes,addfrac

integer*4 deltaN1,deltaN,addNp1,Ntotal

logical flagerror

C

real*8 chspx,lispx,hecha,chalen,charad

integer*4 Nlix,Nchax

logical flagchain

C

integer*4 FD,nx1,nx2

logical flagFD

real*8 x1FD,x2FD

C

C Begin changes of version 39e.

C

logical flagaxi

C

C End changes of version 39e.

C

C

C Begin changes of version 39f.

C

logical flagquar

C

C End changes of version 39f.

C

C

C Begin changes of version 39h.

C

real*8 dbump

C

C End changes of version 39h.

C

write (*,'(A)') 'Output file name?'

Read (*,*) fname

C

C Begin changes of version 39e.

C

write (*,'(a)') 'Axisymmetric cell with no straight section?'

write (*,'(a)') 'yes ...... enter T'

C

read (*,*) flagaxi

C

C End changes of version 39e.

C

C

C Begin changes of version 39f.

C

if (flagaxi) then

C

flagquar = .false.

C

write (*,'(a)') 'Consider only a quarter circular section?'

write (*,'(a)') 'yes ...... enter T'

C

read (*,*) flagquar

C

end if

C

C End changes of version 39f.

C

C

C Begin changes of version 39h.

C

write (*,'(a)') 'Insert a spherical bump obstruction?'

write (*,'(a)') 'yes ... enter bump diameter. no ... enter 0'

C

read (*,*) dbump

C

C End changes of version 39h.

C

CCC

C Initialize Pi and random number:

CCC

C

C SU	pi = ran0(-9)

C SP2	CALL RANDOM_SEED

C GF	pi = ran1(-9)

C

pi = 3.14159265359

C

CCCCCCCCCCCCCCCCCCCCC

C Enter dimensions of the microgravity shear cell

C and the sphere diameters:

CCC

C

C Begin changes of version 39e.

C

308		if (.not.flagaxi) then

C

write (*,'(A)') 'Straight section length?'

Read (*,*) xlen

C

write (*,'(A)') 'Distance between belt and opposite wall?'

Read (*,*) ylen

C

else

C

write (*,'(A)') 'Distance between belt and opposite wall?'

Read (*,*) ylen

C

C If the cell is axisymmetric, make the "dummy" straight section

C length equal to ylen.

C

xlen = ylen

C

end if

C

C End changes of version 39e.

C

write (*,'(A)') 'Straight section width?'

Read (*,*) zlen

C

write (*,'(A)') 'Inner radius of curved section?'

Read (*,*) inrad

C

307	dint1 = 0.0

dint2 = 0.0

dbel = 0.0

dwal = 0.0

C

300	write (*,'(A)') 'Interior sphere diameter for species 1'

Read (*,*) dint1

if (dint1.le.0.0) go to 300

dmin = dint1

dmax = dint1

C

301	write (*,'(A)') 'Interior sphere diameter for species 2'

write (*,'(A)') 'if only one species, enter same diameter'

Read (*,*) dint2

if (dint2.le.0.0) go to 301

if (dint2.gt.dmax) dmax = dint2

if (dint2.lt.dmin) dmin = dint2

C

write (*,'(A)') 'species 1 density is assumed to be one'

306	write (*,'(A)') 'species 2 density?'

read (*,*) dens2

if (dens2.le.0.0) go to 306

C

302	write (*,'(A)') 'proportion of species 1?'

Read (*,*) fracpsd1

if (fracpsd1.lt.0.0.or.fracpsd1.gt.1.0) go to 302

C

if (dint1.eq.dint2.and.dens2.eq.1.0.

&			and.fracpsd1.ne.1.0) then

write (*,'(a)')

&	'single species parameters for multiple species flow...'

go to 307

end if

C

if ( (dint1.ne.dint2.or.dens2.ne.1.0).

&			and.fracpsd1.eq.1.0) then

write (*,'(a)')

&	'single species flow has multiple species parameters...'

go to 307

end if

C

C

303	write (*,'(A)') 'Belt sphere diameter?'

Read (*,*) dbel

if (dbel.le.0.0) go to 303

if (dbel.gt.dmax) dmax = dbel

if (dbel.lt.dmin) dmin = dbel

C

304	write (*,'(A)') 'Wall sphere diameter?'

Read (*,*) dwal

if (dwal.le.0.0) go to 304

if (dwal.gt.dmax) dmax = dwal

if (dwal.lt.dmin) dmin = dwal

C

C Begin changes of version 39h.

C

if (dbump.gt.0.0) then

C

if (dbump.gt.dmax) dmax = dbump

if (dbump.lt.dmin) dmin = dbump

C

end if

C

C End changes of version 39h.

C

C

C Calculate the possible range of search grid cell size:

C

imax = int( dmax/dmin*sqrt(3.) ) + 1

imaxwall = int(imax/2.0 - 0.1)

C

305	write (*,'(A)') 'Cell size?'

write (*,'(2(a,f8.6))') 'enter number between ',

&	dmax/imax,' and ',dmin/sqrt(3.)

write (*,'(a,f8.6)') 'recommended is ',

&	(dmax/imax + dmin/sqrt(3.) ) /2.

read (*,*) cell

if (cell.le.(dmax/imax).or.

&	cell.ge.(dmin/sqrt(3.)) ) go to 305

C

CCC

C Adjust xlen, ylen, zlen and inrad to be multiples of grid cell:

CCC

C

Nxlen = int(xlen/cell)

Nylen = int(ylen/cell)

Nzlen = int(zlen/cell)

Ninrad = int(inrad/cell)

xlen = cell*Nxlen

ylen = cell*Nylen

zlen = cell*Nzlen

inrad = cell*Ninrad

C

CCC

C The outer radius must be compatible with the cell height:

CCC

C

outrad = inrad + ylen

C

C Query whether the flow is FD or not:

C

C Begin changes of version 39e.

C

flagFD = .false.

C

if (.not.flagaxi) then

C

write (*,*) 'Fill in only the fully-developed region?'

write (*,*) 'yes is 1; no is otherwise'

read (*,*) FD

if (FD.eq.1) flagFD=.true.

C

end if

C

C End changes of version 39e.

C

C In case the flow is only FD, then enter the relevant section parameters:

C

if (flagFD) then

C

400	write (*,*) 'enter grid indices of the FD region'

write (*,*) '0 < nx1 < nx2 < ',nxlen

C

write (*,*) 'enter nx1'

read (*,*) nx1

C

write (*,*) 'enter nx2'

read (*,*) nx2

C

if (nx2.le.nx1) then

if (flag) write (*,*) 'nx2 < or = nx1'

go to 400

end if

C

if (nx1.lt.1) then

if (flag) write (*,*) 'nx1 < 1'

go to 400

end if

C

if (nx1.gt.nxlen) then

if (flag) write (*,*) 'nx1 > nxlen'

go to 400

end if

C

if (nx2.lt.1) then

if (flag) write (*,*) 'nx2 < 1'

go to 400

end if

C

if (nx2.gt.nxlen) then

if (flag) write (*,*) 'nx2 > nxlen'

go to 400

end if

C

x1FD = outrad + (nx1 - 1) * cell

x2FD = x1FD + (nx2 - nx1 + 1)*cell

C

else

C

C Verify that there will be enough reinsertion points for escape

C or additional spheres:

C

C Calculate the number of reinsertion points:

C

C

C Begin changes of version 39e.

C

C If the cell is axisymmetric, the following instructions are superfluous,

C since there are only two reinsertion points:

C

if (.not.flagaxi) then

C

C End changes of version 39e.

C

flagerror = .false.

C

nx0 = 1

nxfin = nxlen

nxdomain = nxfin - nx0 + 1

nreinx = nxdomain / (2*imax+1)

C

if (nreinx.lt.1) then

write (*,*) 'nreinx < 1 : make section longer in x-dir:'

write (*,'(a,f10.5)') 'xlen > ',cell*(2*imax+1)

flagerror = .true.

end if

C

ny0 = 1 + max(1+int(dmax/cell),

&				nya+imaxwall)

nyfin = min(nylen - int(dmax/cell),

&				nylen - imaxwall) - 1

nydomain = nyfin - ny0 + 1

nreiny = nydomain / (2*imax+1)

C

if (nreiny.lt.1) then

write (*,*) 'nreiny < 1 : make section longer in y-dir:'

write (*,'(a,f10.5)') 'ylen > ',

&		cell*(2*imax+2*int(dmax/cell)+3)

flagerror = .true.

end if

C

nza = 1

nz0 = nza + imaxwall + 1

nzfin = nzlen - imaxwall - 1

nzdomain = nzfin - nz0 + 1

nreinz = nzdomain / (2*imax+1)

C

if (nreinz.lt.1) then

write (*,*) 'nreinz < 1 : make section longer in z-dir:'

write (*,'(a,f10.5)') 'zlen > ',

&		cell*(2*imax+2*imaxwall+3)

flagerror = .true.

end if

C

if (flagerror) go to 308

C

C

C Begin changes of version 39e.

C

end if

C

C End changes of version 39e.

C

end if

C

CCC

C Report new values of xlen, ylen, zlen and inrad:

CCC

C

write (*,'(a,/,3(a,f10.5),/,a,f10.5,/a)')

&	'Adjusted values of',

&	'xlen = ',xlen,'  ylen = ',ylen,'  zlen = ',zlen,

&	'  inner radius = ',inrad,

&	'OK to continue?  --  yes = 1; no = 0; stop < 0: '

C

read(*,*) OK

C

if (OK.ne.1) then

if (OK.lt.0) stop

go to 305

end if

C

C Rollers location:

C

xc1 = outrad

yc1 = outrad

xc2 = outrad + xlen

yc2 = outrad

C

C Enter velocities of each wall:

C

write (*,'(a)')

&	'Enter wall velocities in arbitrary units.'

write (*,'(a)')

&	'These velocities will be rescaled for unity shear rate.'

444	write (*,'(a)')

&	'Enter inner wall velocity:'

read (*,*) vbelt

write (*,'(a)')

&	'Enter outer wall velocity:'

read (*,*) vwall

if (vbelt.eq.vwall) then

write (*,'(a)')

&	'The two walls should not have the same velocity.'

go to 444

end if

C

C Begin changes of version 39e.

C

flagw = .false.

C

C The following ensures that the outer wall is always uniform if

C the cell is axisymmetric:

C

if (vwall.eq.0.0.and.(.not.flagaxi)) then

C

C End changes of version 39e.

C

write (*,'(a)') 'Uniform bumpiness of outer wall?'

write (*,'(a)') 'Yes - enter 1'

read (*,*) ok

if (ok.eq.1) flagw = .True.

C

end if

C

C Initialize variables:

C

waspx10 = 0.0

waspx20 = 0.0

waspx30 = 0.0

waspx40 = 0.0

C

waspxf1 = 0.0

waspxf2 = 0.0

waspxf3 = 0.0

waspxf4 = 0.0

C

Nwaxf1 = 0

Nwaxf2 = 0

Nwaxf3 = 0

Nwaxf4 = 0

C

C Initialize flags:

C

flagc1 = .true.

flagc2 = .true.

flagc3 = .true.

flagc4 = .true.

flagcb = .true.

C

CCCCCCCCCCCCCCCCCCCCC

C Enter geometry of boundary spheres;

C square packing:

CCC

C

if (vwall.ne.0.0.or.flagw) then

C

write (*,'(A)') 'Proposed wall sphere spacing?'

Read (*,*) waspx

C

C Begin changes of version 39e.

C

C The following ensures that the outer wall only features

C a square array of bumps or cylindrical bumps.  This can be

C changed later to permit hexagonal bumps if necessary.

C

200		if (.not.flagaxi) then

C

write (*,'(A)') 'Hexagonal packing = enter 1'

write (*,'(A)') 'Square = enter 0'

write (*,'(A)') 'Cylinders = enter 2'

Read (*,*) ok

C

else

C

write (*,'(A)') 'Square packing = enter 0'

write (*,'(A)') 'Cylinders = enter 2'

Read (*,*) ok

C

if (ok.ne.0.and.ok.ne.2) go to 200

C

end if

C

C End changes of version 39e.

C

if (ok.eq.1) then

flagb = .true.

else

flagb = .false.

if (ok.eq.2) then

flagc1 = .false.

else

if (ok.ne.0) go to 200

end if

end if

C

If (waspx.lt.dwal) then

write (*,'(a,f10.4)') 'spacing adjusted to ',dwal

waspx = dwal

End if

C

else

C

write (*,'(A)') 'Region 1'

C

201	write (*,'(A)') 'Hexagonal packing = enter 1'

write (*,'(A)') 'Square = enter 0'

write (*,'(A)') 'Cylinders = enter 2'

Read (*,*) ok

C

if (ok.eq.1) then

flagb1 = .true.

else

flagb1 = .false.

if (ok.eq.2) then

flagc1 = .false.

else

if (ok.ne.0) go to 201

end if

end if

C

write (*,'(A)') 'Variable packing = enter 1'

write (*,'(A)') 'Uniform packing = enter other'

Read (*,*) ok

C

if (ok.eq.1) then

flagw1 = .true.

else

flagw1 = .false.

end if

C

if (flagw1) then

C

write (*,'(A)') 'Initial wall sphere spacing?'

Read (*,*) waspx10

C

write (*,'(A)') 'Final wall sphere spacing?'

Read (*,*) waspx1f

C

511	write (*,'(A)') 'fraction of the variable section?'

write (*,'(A)') 'enter number between 0 and 1'

Read (*,*) frac1

if (frac1.lt.0.0.or.frac1.gt.1.0) go to 511

C

If (waspx10.lt.dwal) then

write (*,'(a,f10.4)') 'Initial spacing adjusted to ',dwal

waspx10 = dwal

End if

C

If (waspx1f.lt.dwal) then

write (*,'(a,f10.4)') 'Final spacing adjusted to ',dwal

waspx1f = dwal

End if

C

if (flagc1) then

C

write (*,'(A)')

&	'Average spacing to calculate z-spacing?'

Read (*,*) waspx1

C

If (waspx1.lt.dwal) then

write (*,'(a,f10.4)') 'Average spacing adjusted to ',dwal

waspx1 = dwal

End if

C

end if

C

else

C

write (*,'(A)') 'Proposed wall sphere spacing?'

Read (*,*) waspx1

C

If (waspx1.lt.dwal) then

write (*,'(a,f10.4)') 'Region 1 spacing adjusted to ',dwal

waspx1 = dwal

End if

C

end if

C

write (*,'(A)') 'Region 2'

C

202	write (*,'(A)') 'Hexagonal packing = enter 1'

write (*,'(A)') 'Square = enter 0'

write (*,'(A)') 'Cylinders = enter 2'

Read (*,*) ok

C

if (ok.eq.1) then

flagb2 = .true.

else

flagb2 = .false.

if (ok.eq.2) then

flagc2 = .false.

else

if (ok.ne.0) go to 202

end if

end if

C

write (*,'(A)') 'Variable packing = enter 1'

write (*,'(A)') 'Uniform packing = enter other'

Read (*,*) ok

C

if (ok.eq.1) then

flagw2 = .true.

else

flagw2 = .false.

end if

C

if (flagw2) then

C

write (*,'(A)') 'Initial wall sphere spacing?'

Read (*,*) waspx20

C

write (*,'(A)') 'Final wall sphere spacing?'

Read (*,*) waspx2f

C

512	write (*,'(A)') 'fraction of the variable section?'

write (*,'(A)') 'enter number between 0 and 1'

Read (*,*) frac2

if (frac2.lt.0.0.or.frac2.gt.1.0) go to 512

C

If (waspx20.lt.dwal) then

write (*,'(a,f10.4)') 'Initial spacing adjusted to ',dwal

waspx20 = dwal

End if

C

If (waspx2f.lt.dwal) then

write (*,'(a,f10.4)') 'Final spacing adjusted to ',dwal

waspx2f = dwal

End if

C

if (flagc2) then

C

write (*,'(A)')

&	'Average spacing to calculate z-spacing?'

Read (*,*) waspx2

C

If (waspx2.lt.dwal) then

write (*,'(a,f10.4)') 'Average spacing adjusted to ',dwal

waspx2 = dwal

End if

C

end if

C

else

C

write (*,'(A)') 'Proposed wall sphere spacing?'

Read (*,*) waspx2

C

If (waspx2.lt.dwal) then

write (*,'(a,f10.4)') 'Region 2 spacing adjusted to ',dwal

waspx2 = dwal

End if

C

end if

C

write (*,'(A)') 'Region 3'

C

203	write (*,'(A)') 'Hexagonal packing = enter 1'

write (*,'(A)') 'Square = enter 0'

write (*,'(A)') 'Cylinders = enter 2'

Read (*,*) ok

C

if (ok.eq.1) then

flagb3 = .true.

else

flagb3 = .false.

if (ok.eq.2) then

flagc3 = .false.

else

if (ok.ne.0) go to 203

end if

end if

C

write (*,'(A)') 'Variable packing = enter 1'

write (*,'(A)') 'Uniform packing = enter other'

Read (*,*) ok

C

if (ok.eq.1) then

flagw3 = .true.

else

flagw3 = .false.

end if

C

if (flagw3) then

C

write (*,'(A)') 'Initial wall sphere spacing?'

Read (*,*) waspx30

C

write (*,'(A)') 'Final wall sphere spacing?'

Read (*,*) waspx3f

C

513	write (*,'(A)') 'fraction of the variable section?'

write (*,'(A)') 'enter number between 0 and 1'

Read (*,*) frac3

if (frac3.lt.0.0.or.frac3.gt.1.0) go to 513

C

If (waspx30.lt.dwal) then

write (*,'(a,f10.4)') 'Initial spacing adjusted to ',dwal

waspx30 = dwal

End if

C

If (waspx3f.lt.dwal) then

write (*,'(a,f10.4)') 'Final spacing adjusted to ',dwal

waspx3f = dwal

End if

C

if (flagc3) then

C

write (*,'(A)')

&	'Average spacing to calculate z-spacing?'

Read (*,*) waspx3

C

If (waspx3.lt.dwal) then

write (*,'(a,f10.4)') 'Average spacing adjusted to ',dwal

waspx3 = dwal

End if

C

end if

C

else

C

write (*,'(A)') 'Proposed wall sphere spacing?'

Read (*,*) waspx3

C

If (waspx3.lt.dwal) then

write (*,'(a,f10.4)') 'Region 3 spacing adjusted to ',dwal

waspx3 = dwal

End if

C

end if

C

write (*,'(A)') 'Region 4'

C

204	write (*,'(A)') 'Hexagonal packing = enter 1'

write (*,'(A)') 'Square = enter 0'

write (*,'(A)') 'Cylinders = enter 2'

Read (*,*) ok

C

if (ok.eq.1) then

flagb4 = .true.

else

flagb4 = .false.

if (ok.eq.2) then

flagc4 = .false.

else

if (ok.ne.0) go to 204

end if

end if

C

write (*,'(A)') 'Variable packing = enter 1'

write (*,'(A)') 'Uniform packing = enter other'

Read (*,*) ok

C

if (ok.eq.1) then

flagw4 = .true.

else

flagw4 = .false.

end if

C

if (flagw4) then

C

write (*,'(A)') 'Initial wall sphere spacing?'

Read (*,*) waspx40

C

write (*,'(A)') 'Final wall sphere spacing?'

Read (*,*) waspx4f

C

514	write (*,'(A)') 'fraction of the variable section?'

write (*,'(A)') 'enter number between 0 and 1'

Read (*,*) frac4

if (frac4.lt.0.0.or.frac4.gt.1.0) go to 514

C

If (waspx40.lt.dwal) then

write (*,'(a,f10.4)') 'Initial spacing adjusted to ',dwal

waspx40 = dwal

End if

C

If (waspx4f.lt.dwal) then

write (*,'(a,f10.4)') 'Final spacing adjusted to ',dwal

waspx4f = dwal

End if

C

if (flagc4) then

C

write (*,'(A)')

&	'Average spacing to calculate z-spacing?'

Read (*,*) waspx4

C

If (waspx4.lt.dwal) then

write (*,'(a,f10.4)') 'Average spacing adjusted to ',dwal

waspx4 = dwal

End if

C

end if

C

else

C

write (*,'(A)') 'Proposed wall sphere spacing?'

Read (*,*) waspx4

C

If (waspx4.lt.dwal) then

write (*,'(a,f10.4)') 'Region 4 spacing adjusted to ',dwal

waspx4 = dwal

End if

C

end if

C

end if

C

C Determine here whether there is a chain driving the inner belt

C bumps:

C

C Begin changes of version 39e.

C

C If the cell is axisymmetric, there is no chain:

C

flagchain = .false.

C

if (.not.flagaxi) then

C

write (*,*) 'Is there a chain driving the bumps?'

write (*,*) 'yes, enter 1; no, enter 0'

C

read (*,*) OK

C

if (OK.ne.1) then

flagchain = .false.

else

flagchain = .true.

end if

C

end if

C

C End changes of version 39e.

C

309		continue

C

if (flagchain) then

C

write (*,*) 'enter proposed length of chain link'

read (*,*) chspx

if (chspx.lt.dbel) then

write (*,*) 'chain link smaller than bump diameter'

go to 309

end if

if (chspx.gt.inrad) then

write (*,*) 'chain link greater than sprocket radius'

go to 309

end if

C

310	write (*,*) 'enter proposed bump spacing on link'

read (*,*) lispx

if (lispx.gt.chspx) then

write (*,*) 'bump spacing larger than chain link'

go to 310

end if

if (lispx.lt.dbel) then

write (*,*) 'bump spacing smaller than bump diameter'

go to 310

end if

C

206	write (*,'(A)') 'square packing = enter 0'

write (*,'(A)') 'cylinders = enter 2'

write (*,'(A)') 'stop, enter <0'

Read (*,*) ok

if (ok.lt.0) stop

C

C No hexagonal packing on chain link:

C

flagbb = .false.

C

if (ok.eq.2) then

flagcb = .false.

else

flagcb = .true.

if (ok.ne.0) go to 206

end if

C

207	write (*,*)

&	'enter height of bump centers above chain link'

read (*,*) hecha

if (hecha.lt.0.0) then

write (*,*) 'bump height is negative'

go to 207

end if

C

else

C

write (*,'(A)') 'Proposed belt sphere spacing?'

Read (*,*) bespx

hecha = 0.0

C

C Begin changes of version 39e.

C

C The following ensures that the inner wall only features

C a square array of bumps or cylindrical bumps.  This can be

C changed later to permit hexagonal bumps if necessary.

C

205		if (.not.flagaxi) then

C

write (*,'(A)') 'Hexagonal packing = enter 1'

write (*,'(A)') 'Square = enter 0'

write (*,'(A)') 'Cylinders = enter 2'

Read (*,*) ok

C

else

C

write (*,'(A)') 'Square packing = enter 0'

write (*,'(A)') 'Cylinders = enter 2'

Read (*,*) ok

C

if (ok.ne.0.and.ok.ne.2) go to 205

C

end if

C

C End changes of version 39e.

C

if (ok.eq.1) then

flagbb = .true.

else

flagbb = .false.

if (ok.eq.2) then

flagcb = .false.

else

if (ok.ne.0) go to 205

end if

end if

C

If (bespx.lt.dbel) then

write (*,'(a,f10.4)') 'spacing adjusted to ',dbel

bespx = dbel

End if

C

end if

C

C

CCCCCCCCCCCCCCCCCCCCC

C Adjust the spacing in both directions to obtain a

C periodic pattern on the wall and the belt:

C

C Calculate the total wall and belt length:

C

C Begin changes of version 39e.

C

if (.not.flagaxi) then

C

walen = 2.*pi*outrad + 2.*xlen

belen = 2.*pi*inrad + 2.*xlen

C

else

C

walen = 2.*pi*outrad

belen = 2.*pi*inrad

C

end if

C

C End changes of version 39e.

C

if (vwall.ne.0.0.or.flagw) then

C

C Find the total number of boundary spheres in both directions;

C Begin with the z-direction:

C

if (flagb) then

Nwaz = int( (zlen-dwal) /waspx*2./sqrt(3.) )

waspz = (zlen-dwal)/Nwaz

C

write (*,'(a,f10.4)')

&	'spacing across the uniform hexagonal wall adjusted to ',

&	waspz

C

Nwax = int( walen /waspx*2.)

C

C The following statement ensures that Nwax is always even

C for a periodic wall; this ensures continuity at the

C periodic boundary:

C

if (mod(Nwax,2).eq.1) Nwax = Nwax - 1

C

waspx = 2.*walen / Nwax

C

write (*,'(a,f10.4)')

&	'spacing along the hexagonal periodic wall adjusted to ',

&	waspx

C

else

C

C Square packing; cylinders do not apply:

C

if (flagc1) then

C

Nwaz = int(zlen/waspx)

waspz = zlen/Nwaz

C

write (*,'(a,f10.4)')

&	'spacing across the square uniform wall adjusted to ',

&	waspz

C

end if

C

Nwax = int(walen/waspx)

waspx = walen/Nwax

C

write (*,'(a,f10.4)')

&	'spacing along the square uniform wall adjusted to ',

&	waspx

C

end if

C

C Register dummy values of Nwax1->4 and Nwaz1->4:

C

flagb1 = flagb

C

Nwax1 = Nwax

Nwaz1 = Nwaz

Nwax2 = 0

Nwaz2 = 0

Nwax3 = 0

Nwaz3 = 0

Nwax4 = 0

Nwaz4 = 0

C

else

C

C Region 1:

C

if (flagb1) then

C

C Hexagonal packing first:

C

Nwaz1 = int( (zlen-dwal) /waspx1*2./sqrt(3.) )

waspz1 = (zlen-dwal)/Nwaz1

C

write (*,'(a,f10.4)')

& 'spacing across the hexagonal wall in region 1 adjusted to ',

&	waspz1

C

if (flagw1) then

C

C Variable spacing first:

C

C Calculate the number of sphere columns in the x-direction located in

C the fixed-spacing section of region 1:

C

Nwaxf1 = int( 2.*(1.-frac1)*(xlen-dwal) / waspx1f)

C

C In the event that the fixed-spacing section has no sphere,

C make the "fixed spacing" zero and the fraction identically

C one...

C

if (Nwaxf1.eq.0) then

frac1 = 1.0

waspxf1 = 0.0

else

C

C ... otherwise, readjust the spacing to fit an integer number of

C      columns in the fixed-spacing section:

C

waspxf1 = 2.*(1.-frac1)*(xlen-dwal)/Nwaxf1

end if

C

C Calculate the number of sphere columns in the x-direction located

C in the variable-spacing section of region 1:

C

Nwax1 = int( 4.*frac1*(xlen-dwal) / (waspx10+waspx1f) )

C

C In the event that the variable-spacing section has no sphere,

C warn that the spacing is in fact fixed and calculate it

C accordingly...

C

if (Nwax1.eq.0) then

waspx1 = waspx1f

waspx10 = 0.0

Nwax1 = int( (xlen-dwal) /waspx1*2.)

waspx1 = 2.*(xlen-dwal) / Nwax1

Nwax1 = Nwax1 + 1

write (*,'(a)') 'fixed spacing in fact...'

write (*,'(a,f10.4)')

&	'spacing along the hexagonal wall in region 1 adjusted to ',

&	waspx1

C

else

C

C ... otherwise, readjust the initial spacing to fit an integer number

C      of colums in the variable section:

C

fact = 4.*frac1*(xlen-dwal) / Nwax1 / (waspx10+waspx1f)

waspx10 = fact*waspx10

Nwax1 = Nwax1 + 1

C

write (*,'(a,f10.4)')

&	'Initial spacing along region 1 adjusted to ',

&	waspx10

C

end if

C

else

C

C fixed spacing:

C

Nwax1 = int( (xlen-dwal) /waspx1*2.)

waspx1 = 2.*(xlen-dwal) / Nwax1

Nwax1 = Nwax1 + 1

C

write (*,'(a,f10.4)')

&	'spacing along the hexagonal wall in region 1 adjusted to ',

&	waspx1

C

end if

C

else

C

C Square packing:

C

if (flagc1) then

C

C Cylinders do not apply to spacing across:

C

Nwaz1 = int(zlen/waspx1)

waspz1 = zlen/Nwaz1

C

write (*,'(a,f10.4)')

& 'Sphere spacing across the wall in region 1 adjusted to ',

&	waspz1

C

C

end if

if (flagw1) then

C

C variable spacing:

C

C Calculate the number of sphere columns in the x-direction located in

C the fixed-spacing section of region 1:

C

Nwaxf1 = int( (1.-frac1)*xlen / waspx1f)

C

C In the event that the fixed-spacing section has no sphere,

C make the "fixed spacing" zero and the fraction identically

C one...

C

if (Nwaxf1.eq.0) then

frac1 = 1.0

waspxf1 = 0.0

else

C

C ... otherwise, readjust the spacing to fit an integer number of

C      columns in the fixed-spacing section:

C

waspxf1 = (1.-frac1)*xlen /Nwaxf1

end if

C

C Calculate the number of sphere columns in the x-direction located

C in the variable-spacing section of region 1:

C

Nwax1 = int( 2.*frac1*xlen / (waspx10+waspx1f) )

C

C In the event that the variable-spacing section has no sphere,

C warn that the spacing is in fact fixed and calculate it

C accordingly...

C

if (Nwax1.eq.0) then

waspx1 = waspx1f

waspx10 = 0.0

Nwax1 = int( xlen /waspx1 )

waspx1 = xlen / Nwax1

write (*,'(a)') 'fixed spacing in fact...'

write (*,'(a,f10.4)')

&	'spacing along the square wall in region 1 adjusted to ',

&	waspx1

C

else

C

C ... otherwise, readjust the initial spacing to fit an integer number

C      of colums in the variable section:

C

fact = 2.*frac1*xlen / Nwax1 / (waspx10+waspx1f)

waspx10 = fact*waspx10

C

write (*,'(a,f10.4)')

&	'Initial spacing along region 1 adjusted to ',

&	waspx10

C

end if

C

else

C

C fixed spacing:

C

Nwax1 = int(xlen/waspx1)

waspx1 = xlen/Nwax1

C

write (*,'(a,f10.4)')

&	'spacing along the wall in region 1 adjusted to ',

&	waspx1

C

end if

C

end if

C

C Region 2:

C

if (flagb2) then

C

Nwaz2 = int( (zlen-dwal) /waspx2*2./sqrt(3.) )

waspz2 = (zlen-dwal)/Nwaz2

C

write (*,'(a,f10.4)')

& 'spacing across the hexagonal wall in region 2 adjusted to ',

&	waspz2

C

if (flagw2) then

C

Nwaxf2 = int( 2.*(1.-frac2)*(pi*outrad-dwal) /

&				waspx2f)

if (Nwaxf2.eq.0) then

frac2 = 1.0

waspxf2 = 0.0

else

waspxf2 = 2.*(1.-frac2)*

&		(pi*outrad-dwal)/Nwaxf2

end if

Nwax2 = int( 4.*frac2*(pi*outrad-dwal) /

&			(waspx20+waspx2f) )

if (Nwax2.eq.0) then

waspx2 = waspx2f

waspx20 = 0.0

Nwax2 = int( (pi*outrad-dwal) /waspx2*2.)

waspx2 = 2.*(pi*outrad-dwal) / Nwax2

Nwax2 = Nwax2 + 1

write (*,'(a)') 'fixed spacing in fact...'

write (*,'(a,f10.4)')

&	'spacing along the hexagonal wall in region 2 adjusted to ',

&	waspx2

C

else

C

fact = 4.*frac2*(pi*outrad-dwal) / Nwax2 /

&			(waspx20+waspx2f)

waspx20 = fact*waspx20

Nwax2 = Nwax2 + 1

C

write (*,'(a,f10.4)')

&	'Initial spacing along region 2 adjusted to ',

&	waspx20

C

end if

C

else

C

Nwax2 = int( (pi*outrad-dwal) /waspx2*2.)

waspx2 = 2.*(pi*outrad-dwal) / Nwax2

Nwax2 = Nwax2 + 1

C

write (*,'(a,f10.4)')

&	'spacing along the hexagonal wall in region 2 adjusted to ',

&	waspx2

C

end if

C

else

C

C Square packing; cylinders do not apply:

C

if (flagc2) then

C

Nwaz2 = int(zlen/waspx2)

waspz2 = zlen/Nwaz2

C

write (*,'(a,f10.4)')

& 'Sphere spacing across the wall in region 2 adjusted to ',

&	waspz2

C

end if

C

if (flagw2) then

C

Nwaxf2 = int( (1.-frac2)*pi*outrad / waspx2f)

if (Nwaxf2.eq.0) then

frac2 = 1.0

waspxf2 = 0.0

else

waspxf2 = (1.-frac2)*pi*outrad/Nwaxf2

end if

C

Nwax2 = int( 2.*frac2*pi*outrad/

&			(waspx20+waspx2f) )

if (Nwax2.eq.0) then

waspx2 = waspx2f

waspx20 = 0.0

Nwax2 = int( pi*outrad / waspx2 )

waspx2 = pi*outrad / Nwax2

write (*,'(a)') 'fixed spacing in fact...'

write (*,'(a,f10.4)')

&	'spacing along the square wall in region 2 adjusted to ',

&	waspx2

C

else

fact = 2.*frac2*pi*outrad/ Nwax2 /

&		(waspx20+waspx2f)

waspx20 = fact*waspx20

C

write (*,'(a,f10.4)')

&	'Initial spacing along region 2 adjusted to ',

&	waspx20

C

end if

C

else

C

Nwax2 = int(pi*outrad/waspx2)

waspx2 = pi*outrad/Nwax2

C

write (*,'(a,f10.4)')

&	'spacing along the wall in region 2 adjusted to ',

&	waspx2

C

end if

C

end if

C

C Region 3:

C

if (flagb3) then

C

Nwaz3 = int( (zlen-dwal) /waspx3*2./sqrt(3.) )

waspz3 = (zlen-dwal)/Nwaz3

C

write (*,'(a,f10.4)')

& 'spacing across the hexagonal wall in region 3 adjusted to ',

&	waspz3

C

if (flagw3) then

C

Nwaxf3 = int( 2.*(1.-frac3)*(xlen-dwal) / waspx3f)

C

if (Nwaxf3.eq.0) then

frac3 = 1.0

waspxf3 = 0.0

else

waspxf3 = 2.*(1.-frac3)*(xlen-dwal)/Nwaxf3

end if

Nwax3 = int( 4.*frac3*(xlen-dwal) /

&			(waspx30+waspx3f) )

if (Nwax3.eq.0) then

waspx3 = waspx3f

waspx30 = 0.0

Nwax3 = int( (xlen-dwal) /waspx3*2.)

waspx3 = 2.*(xlen-dwal) / Nwax3

Nwax3 = Nwax3 + 1

write (*,'(a)') 'fixed spacing in fact...'

write (*,'(a,f10.4)')

&	'spacing along the hexagonal wall in region 3 adjusted to ',

&	waspx3

C

else

fact = 4.*frac3*(xlen-dwal) / Nwax3 /

&		(waspx30+waspx3f)

waspx30 = fact*waspx30

Nwax3 = Nwax3 + 1

C

write (*,'(a,f10.4)')

&	'Initial spacing along region 3 adjusted to ',

&	waspx30

C

end if

C

else

C

Nwax3 = int( (xlen-dwal) /waspx3*2.)

waspx3 = 2.*(xlen-dwal) / Nwax3

Nwax3 = Nwax3 + 1

C

write (*,'(a,f10.4)')

&	'spacing along the hexagonal wall in region 3 adjusted to ',

&	waspx3

C

end if

C

else

C

C Square packing; cylinders do not apply:

C

if (flagc3) then

C

Nwaz3 = int(zlen/waspx3)

waspz3 = zlen/Nwaz3

C

write (*,'(a,f10.4)')

& 'Sphere spacing across the wall in region 3 adjusted to ',

&	waspz3

C

end if

C

if (flagw3) then

C

Nwaxf3 = int( (1.-frac3)*xlen / waspx3f)

if (Nwaxf3.eq.0) then

frac3 = 1.0

waspxf3 = 0.0

else

waspxf3 = (1.-frac3)*xlen /Nwaxf3

end if

C

Nwax3 = int( 2.*frac3*xlen / (waspx30+waspx3f) )

if (Nwax3.eq.0) then

waspx3 = waspx3f

waspx30 = 0.0

Nwax3 = int( xlen /waspx3 )

waspx3 = xlen / Nwax3

write (*,'(a)') 'fixed spacing in fact...'

write (*,'(a,f10.4)')

&	'spacing along the square wall in region 3 adjusted to ',

&	waspx3

C

else

fact = 2.*frac3*xlen / Nwax3 / (waspx30+waspx3f)

waspx30 = fact*waspx30

C

write (*,'(a,f10.4)')

&	'Initial spacing along region 3 adjusted to ',

&	waspx30

C

end if

C

else

C

Nwax3 = int(xlen/waspx3)

waspx3 = xlen/Nwax3

C

write (*,'(a,f10.4)')

&	'spacing along the wall in region 3 adjusted to ',

&	waspx3

C

end if

C

end if

C

C Region 4:

C

if (flagb4) then

C

Nwaz4 = int( (zlen-dwal) /waspx4*2./sqrt(3.) )

waspz4 = (zlen-dwal)/Nwaz4

C

write (*,'(a,f10.4)')

& 'spacing across the hexagonal wall in region 4 adjusted to ',

&	waspz4

C

if (flagw4) then

C

Nwaxf4 = int( 2.*(1.-frac4)*(pi*outrad-dwal) /

&				waspx4f)

if (Nwaxf4.eq.0) then

frac4 = 1.0

waspxf4 = 0.0

else

waspxf4 = 2.*(1.-frac4)*

&		(pi*outrad-dwal)/Nwaxf4

end if

Nwax4 = int( 4.*frac4*(pi*outrad-dwal) /

&			(waspx40+waspx4f) )

if (Nwax4.eq.0) then

waspx4 = waspx4f

waspx40 = 0.0

Nwax4 = int( (pi*outrad-dwal) /waspx4*2.)

waspx4 = 2.*(pi*outrad-dwal) / Nwax4

Nwax4 = Nwax4 + 1

write (*,'(a)') 'fixed spacing in fact...'

write (*,'(a,f10.4)')

&	'spacing along the hexagonal wall in region 4 adjusted to ',

&	waspx4

C

else

fact = 4.*frac4*(pi*outrad-dwal) / Nwax4 /

&			(waspx40+waspx4f)

waspx40 = fact*waspx40

Nwax4 = Nwax4 + 1

C

write (*,'(a,f10.4)')

&	'Initial spacing along region 4 adjusted to ',

&	waspx40

C

end if

C

else

C

Nwax4 = int( (pi*outrad-dwal) /waspx4*2.)

waspx4 = 2.*(pi*outrad-dwal) / Nwax4

Nwax4 = Nwax4 + 1

C

write (*,'(a,f10.4)')

&	'spacing along the hexagonal wall in region 4 adjusted to ',

&	waspx4

C

end if

C

else

C

C Square packing; cylinders do not apply:

C

if (flagc4) then

C

Nwaz4 = int(zlen/waspx4)

waspz4 = zlen/Nwaz4

C

write (*,'(a,f10.4)')

& 'Sphere spacing across the wall in region 4 adjusted to ',

&	waspz4

C

end if

C

if (flagw4) then

C

Nwaxf4 = int( (1.-frac4)*pi*outrad / waspx4f)

if (Nwaxf4.eq.0) then

frac4 = 1.0

waspxf4 = 0.0

else

waspxf4 = (1.-frac4)*pi*outrad/Nwaxf4

end if

C

Nwax4 = int( 2.*frac4*pi*outrad/

&			(waspx40+waspx4f) )

if (Nwax4.eq.0) then

waspx4 = waspx4f

waspx40 = 0.0

Nwax4 = int( pi*outrad / waspx4 )

waspx4 = pi*outrad / Nwax4

write (*,'(a)') 'fixed spacing in fact...'

write (*,'(a,f10.4)')

&	'spacing along the square wall in region 4 adjusted to ',

&	waspx4

C

else

fact = 2.*frac4*pi*outrad/ Nwax4 /

&			(waspx40+waspx4f)

waspx40 = fact*waspx40

C

write (*,'(a,f10.4)')

&	'Initial spacing along region 4 adjusted to ',

&	waspx40

C

end if

C

else

C

Nwax4 = int(pi*outrad/waspx4)

waspx4 = pi*outrad/Nwax4

C

write (*,'(a,f10.4)')

&	'spacing along the wall in region 4 adjusted to ',

&	waspx4

C

end if

C

end if

C

end if

C

CCCC

C Inner wall:

C

if (flagbb) then

Nbez = int( (zlen-dbel) /bespx*2./sqrt(3.) )

bespz = (zlen-dbel)/Nbez

C

write (*,'(a,f10.4)')

&	'spacing across the hexagonal belt adjusted to ',

&	bespz

C

Nbex = int( belen /bespx*2.)

C

C The following statement ensures that Nbex is always even

C for a periodic belt; this ensures continuity at the

C periodic boundary:

C

if (mod(Nbex,2).eq.1) Nbex = Nbex - 1

C

bespx = 2.*belen / Nbex

C

write (*,'(a,f10.4)')

&	'spacing along the hexagonal periodic belt adjusted to ',

&	bespx

C

else

C

C Square packing; cylinders do not apply:

C

if (flagcb) then

C

if (.not.flagchain) then

C

Nbez = int(zlen/bespx)

bespz = zlen/Nbez

C

write (*,'(a,f10.4)')

&	'spacing across the square belt adjusted to ',

&	bespz

C

else

C

Nbez = int(zlen/lispx)

bespz = zlen/Nbez

C

end if

C

end if

C

if (.not.flagchain) then

C

Nbex = int(belen/bespx)

bespx = belen/Nbex

Nlix = 0

C

write (*,'(a,f10.4)')

&	'spacing along the square belt adjusted to ',

&	bespx

C

else

C

charad = inrad-hecha

chalen = 2.*pi*charad + 2.*xlen

Nchax = int(chalen / chspx)

chspx = chalen / Nchax

write (*,'(a,f10.4)')

&	'length of chain link adjusted to ',chspx

C

Nlix = int(chspx/lispx)

if (Nlix.le.0) then

write (*,*) 'no bumps on chain links'

go to 309

end if

lispx = chspx / Nlix

write (*,'(i4,a,f10.4)') Nlix,

&	' bumps on each link; spacing adjusted to ',

&			lispx

C

end if

end if

C

CCCCCCCCCCCCCCCCCCCCC

C Calculate interior variables:

C

if (ylen.le.(max(dint1,dint2)+

&		(dwal+dbel)/2.) ) then

write (*,'(a)') 'gap too small'

stop

end if

C

222	continue

C

write (*,'(A)') 'Desired sphere concentration?'

Read (*,*) con

condes = con

C

C Calculate the volume available for interior spheres

C and the equivalent sphere number to be fitted in

C at the desired concentration:

C

volume =

&	2.*xlen*(ylen-dwal/2.-dbel/2.)*zlen +

&	pi*( (outrad-dwal/2.)**2 -

&		(inrad+dbel/2.)**2 )*zlen

C

C For FD flow filling, the volume is smaller:

C

if (flagFD) volume = (x2FD-x1FD)*zlen*(ylen-dwal/2.-dbel/2.)

C

C Begin changes of version 39e.

C

C For axisymmetric cells, the actual volume is only that of the two curved

C sections:

C

if (flagaxi) volume = pi*( (outrad-dwal/2.)**2 -

&		(inrad+dbel/2.)**2 )*zlen

C

C End changes of version 39e.

C

C

C Begin changes of version 39f.

C

if (flagquar) volume = volume / 4.

C

C End changes of version 39f.

C

Ndesired = int(condes*volume/pi*6/

&	(fracpsd1*dint1**3 + (1.-fracpsd1)*dint2**3) )

C

write (*,'(a,i8)')

&	'Desired number of interior spheres = ',

&	Ndesired

C

C Estimate the requisite dilatation of the hexagonal packing;

C take a safety factor to account for the circular

C geometry:

C

write (*,'(A,/,A)')

&	'Safety factor for hexagonal dilatation?',

&	'Enter  0 < factor < 1:'

Read (*,*) safety

C

dil = (pi/6.*sqrt(2.)/con)**(1./3.)

dil = 1.+safety*(dil - 1.)

C

C Calculate the expanded hexagonal spacing of interior

C spheres:

C

dx = dil*max(dint1,dint2)

dy = dil*max(dint1,dint2)*sqrt(2./3.)

dz = dil*max(dint1,dint2)*sqrt(3.)/2.

C

C Write the spacing in three directions:

C

write (*,'(3(a,f10.5,/))')

&	'dx = ',dx,

&	'dy = ',dy,

&	'dz = ',dz

C

CCC

C Enter initial agitation:

C

if (dx.ne.max(dint1,dint2)) then

write (*,'(A,/,a,f15.10)') ' ? agitation to initiate collisions?',

&      'recommended is 0.1*(2dy-d)/(dx-d) =  ',

&      0.1*( 2.*dy - max(dint1,dint2) ) /

&      ( dx - max(dint1,dint2) )

else

write (*,'(A)') ' ? agitation to initiate collisions?'

end if

C

Read (*,*) std

C

CCC

C critical radii around the rollers; center of spheres should

C be within these to belong to the domain:

CCC

C

Rcriti = inrad + (dbel+max(dint1,dint2))/2.

Rcrito = outrad - (dwal+max(dint1,dint2))/2.

C

CCC

C max values of x,y,z beyond which no center can be found:

CCC

C

C The statements can be made more sophisticated than the

C following if desired....

C

xmax = xlen + 2.*outrad -

&      	(dwal+max(dint1,dint2))/2.

C

C For filling FD flows, the last possible value of x is at the end

C of the FD region minus a hair to avoid losing spheres

C just at the end of the FD region (see subroutine gridfl):

C

if (flagFD) xmax = x2FD - max(dint1,dint2)*0.001

C

C and the last value of y is ylen to avoid filling spheres in region III:

C

ymax = 2.*outrad - (dwal+

&      	max(dint1,dint2))/2.

C

if (flagFD) ymax = ylen - (dwal+

&      	max(dint1,dint2))/2.

C

zmax = zlen - max(dint1,dint2)/2.

C

CCC

C Rescale wall velocities; the shear rate is hardwired to +/-1:

CCC

C

factor = ylen/abs(vbelt-vwall)

vbelt = vbelt*factor

vwall = vwall*factor

C

CCCCCCCCCCCCCCCCCCCCC

C Fill in interior spheres:

C

C Position of the first interior particle is the following + dx:

C

xp = (max(dint1,dint2)+dwal)/2.

C

C For filling FD flows, the first value of x is at the edge of the FD region:

C

if (flagFD) xp = x1FD

C

yp = (dwal + max(dint1,dint2) )/2.

zp = max(dint1,dint2)/2.

C

C Initialize counters of particles in the x-direction (Nx),

C of particle rows in the z-direction (Nz), of particle

C layers in the y-direction (Ny) and of the total number

C of authentic particles

C

Nx = 0

Ny = 1

Nz = 1

Np = 0

C

flag = .true.

C

CCCCCCCCCCCCCCCCCCCCC

C     Fill x, then z, then y.  Main Loop:

C

Do While (flag)

C

xp = xp + dx

Nx = Nx+1

C

CCCCCCCCCCCC

C Test whether the sphere belongs to the shear cell:

C

C Verify that the sphere has the proper value of z:

C

if (zp.lt.(max(dint1,dint2)/2.).

&	or.zp.gt.zmax) go to 3

C

C Distinguish first whether the center belongs to the straight

C section or to the curved section:

C

C Left curved region first:

C

if (xp.le.outrad) then

C

C Begin changes of version 39f.

C

if (flagquar) go to 3

C

C End changes of version 39f.

C

dist = sqrt( (xp - xc1)**2 + (yp - yc1)**2 )

C

if (dist.ge.Rcriti.and.dist.le.Rcrito) then

C

C Calculate initial velocity:

C

vel = (vbelt-vwall)*(1.- (dist-inrad)/ylen) + vwall

theta = acos( (yp - yc1)/dist )

Up = - vel*cos(theta)

Vp = - vel*sin(theta)

C

go to 2

else

go to 3

end if

C

end if

C

C Straight region next:

C

if (xp.gt.outrad.and.xp.le.outrad+xlen) then

C

C Begin changes of version 39e.

C

C For axisymmetric cells, there are no spheres in the straight regions:

C

if (flagaxi) go to 3

C

C End changes of version 39e.

C

C test whether the sphere belongs to the lower straight cell:

C

test1 = (yp - (dwal+max(dint1,dint2) )/2.)*

&		(yp - (ylen-(dbel+max(dint1,dint2)

&				)/2.) )

C

C test whether the sphere belongs to the upper straight cell:

C

test2 =

&	(yp - (ylen+2.*inrad+(dbel+

&	max(dint1,dint2))/2.))*

&	(yp - (2.*outrad-(dwal+

&	max(dint1,dint2))/2.) )

C

C In case the flow is FD, make sure that no sphere with x≥x2FD

C is registered, since it won't belong to the FD region:

C

if (xp.gt.xmax) then

test1 = +1.

test2 = +1.

end if

C

if (test1.le.0.0.or.test2.le.0.0) then

C

C Calculate initial velocity of the lower cell; add a fluctuating

C component to Vp:

C

if (test1.le.0.0) then

vel = (vbelt-vwall)*(1.- (ylen-yp)/ylen) + vwall

Up = vel

C

C SU	randum = ran0(1)

C SP2	CALL RANDOM_NUMBER(randum)

C GF	randum = ran1(1)

C

Vp = sign((vbelt-vwall)*std,0.5-randum)

end if

C

C Calculate initial velocity of the upper cell; add a fluctuating

C component to Vp:

C

if (test2.le.0.0) then

vel = (vbelt-vwall)*(1.- (yp-ylen-2.*inrad)/ylen) + vwall

Up = - vel

C

C SU	randum = ran0(1)

C SP2	CALL RANDOM_NUMBER(randum)

C GF	randum = ran1(1)

C

Vp = sign((vbelt-vwall)*std,0.5-randum)

end if

C

go to 2

else

go to 3

end if

C

end if

C

C Right curved region finally:

C

if (xp.gt.outrad+xlen) then

C

dist = sqrt( (xp - xc2)**2 + (yp - yc2)**2 )

C

C Begin changes of version 39f.

C

if (flagquar.and.yp.gt.yc2) go to 3

C

C End changes of version 39f.

C

if (dist.ge.Rcriti.and.dist.le.Rcrito) then

C

C Calculate initial velocity:

C

vel = (vbelt-vwall)*(1.- (dist-inrad)/ylen) + vwall

theta = acos( (yc2 - yp)/dist )

Up = + vel*cos(theta)

Vp = + vel*sin(theta)

C

go to 2

else

go to 3

end if

C

end if

C

C TEST

C

write (*,'(a)') 'this statement should not be reached'

C

C TEST

C

C

CCC

C Case where the sphere belongs to the cell:

C

2	continue

C

Np = Np + 1

C

C Guard against excessive number of interior spheres:

C

if (Np.gt.NN) then

write (*,'(a,/,a,i8)')

&	'Excessive number of interior spheres',

&	'number is ',Np

stop

end if

C

x(Np) = xp

y(Np) = yp

z(Np) = zp

C

u(Np) = Up

v(Np) = Vp

w(Np) = 0.0

C

CCC

C Case where the sphere does not belong to the cell:

C

3	continue

C

C End of authentification of spheres.

C

CCCCCCCCCCCC

C If the row of particles is complete in the x-direction,

C reset the counters, move to the next row, and calculate

C the position of the first particle in the next row:

C

if (xp.gt.xmax) then

Nx = 0

Nz = Nz+1

C

C Note: particles are staggered in two consecutive rows

C according to the hexagonal close-pack arrangement:

C

if (mod(Ny,2) .eq. 1) then

xp = (dwal+max(dint1,dint2))/2. -

&      0.5*mod(Nz-1,2)*dx        ! odd layers in y

C

C In FD flow, the first sphere is at the edge of the FD region;

C the hexagonal packing is also staggered to

C the right of the left face of the FD region to avoid losing spheres:

C

if (flagFD) xp = x1FD +

&      0.5*mod(Nz-1,2)*dx        ! odd layers in y

C

else

xp = (dwal+max(dint1,dint2))/2. -

&      0.5*mod(Nz,2)*dx          ! even layers in y

C

if (flagFD) xp = x1FD +

&      0.5*mod(Nz,2)*dx          ! even layers in y

C

end if

C

zp = zp+dz

end if

C

C If all rows are complete in a given layer, then reset also the

C Nz counter, and recalculate the position of the first particle

C in the first row of the next y-layer:

C

if (zp.gt.zmax) then

Ny = Ny+1

Nz = 1

C

yp = yp+dy

C

CCCCCCCCCCCCCC

C If the particles 'leave' in the y-direction,

C or if the desired number of spheres has been reached,

C stop the filling procedure:

CCCCCCCCCCCCCC

C

if (yp.gt.ymax.or.Np.ge.Ndesired) flag = .false.

C

C Note: particles are staggered in two consecutive layers

C according to the hexagonal close-pack arrangement:

C

if (mod(Ny,2) .eq. 1) then

xp = (dwal+max(dint1,dint2))/2.	! odd layers

C

C In FD flow, the first sphere is at the edge of the FD region;

C the hexagonal packing is also staggered to

C the right of the left face of the FD region to avoid losing spheres:

C

if (flagFD) xp = x1FD	! odd layers

C

else

xp = (dwal+max(dint1,dint2))/2. -

&      0.5*dx					! even layers

C

if (flagFD) xp = x1FD +

&      0.5*dx					! even layers

C

end if

C

zp = max(dint1,dint2)/2. + mod(Ny-1,2)*dz/3.0

end if

C

CCCCCCCCCCCCCCCCCCCCC

C End of main loop:

C

End Do

C

CCCCCCCCCCCCCCCCCCCCC

C Case where the granular mass has a bidisperse size

C distribution:

C

C Assign radius to each sphere according to the desired

C distribution:

C

C Find actual number of spheres of species 1:

C

Npp1 = int(Np*fracpsd1)

C

C Initialize species type:

C

do i = 1,NN

flagspec(i) = .false.

end do

C

Np1 = 0

i = 0

C

C In the event that the desired number of species 1 spheres

C has not yet been reached, do as many passes through

C the set as necessary:

C

100	i = i+1

C

if (i.eq.Np+1) i = 1

C

C Species 1 spheres are assigned to randomly chosen indices

C over the entire set of spheres:

C

C

C SU	randum = ran0(1)

C SP2	CALL RANDOM_NUMBER(randum)

C GF	randum = ran1(1)

C

if (.not.flagspec(i).and.randum.le.fracpsd1) then

C

Np1 = Np1 + 1

flagspec(i) = .true.

C

end if

C

C continue until the desired number of test spheres has

C been reached:

C

if (Np1.lt.Npp1) go to 100

C

C Write global results:

C

con = pi/6./volume*( Np1*dint1**3 +

&			(Np - Np1)*dint2**3 )

C

if (fracpsd1.ne.1.0

&	.and.con.ge.condes) write (*,'(a,f10.7)')

&	'The actual fraction of species 1 is ',

&	float(Np1)/float(Np)

C

C Write resulting concentration:

C

write (*,'(a,f10.5)')

&	'current volume fraction = ',con

C

if (con.ge.condes.or.flagFD) then

C

write (*,'(4(a,/))')

&	'OK to proceed?',

&	'yes = 1',

&	'no = other positive integer',

&	'negative to stop'

C

read (*,*) OK

if (OK.lt.0) stop

C

if (OK.ne.1) then

do i = 1,NN

x(i) = 0.0

y(i) = 0.0

z(i) = 0.0

flagspec(i) = .false.

u(i) = 0.0

v(i) = 0.0

w(i) = 0.0

end do

go to 222

end if

C

else

C

write (*,'(5(a,/))')

&	'OK to proceed?',

&	'yes and ignore concentration discrepancy = 1',

&	'fill it to the desired concentration = 2',

&	'specify new concentration = 0 or other positive integer',

&	'negative to stop'

C

read (*,*) OK

C

deltaN = 0

deltaN1 = 0

C

C Stop the program:

C

if (OK.lt.0) stop

C

C Ignore the smaller concentration than desired:

C

if (OK.eq.1) go to 555

C

C Correct the smaller concentration by adding spheres to

C the "purgatory FIFO:

C

if (OK.eq.2) then

C

C Put additional spheres in the "purgatory" FIFO for later

C insertion by the McGSCv15.f-7/17/96 code.

C

C Calculate the additional number of spheres for each species:

C

deltaN1 = int(fracpsd1*Ndesired) - Np1

deltaN = Ndesired - Np

C

if (Ndesired.gt.NN) then

write (*,*)

&	'Excessive number of interior spheres desired'

stop

end if

C

addfrac = float(deltaN1) / float(deltaN)

C

addNp1 = 0

i = 0

101	i = i+1

C

if (i.eq.deltaN+1) i = 1

C

C Species 1 spheres are assigned to randomly chosen indices

C over the entire set of spheres:

C

C

C SU	randum = ran0(1)

C SP2	CALL RANDOM_NUMBER(randum)

C GF	randum = ran1(1)

C

if (.not.flagspec(i+Np).and.randum.le.addfrac) then

C

addNp1 = addNp1 + 1

C

flagspec(i+Np) = .true.

C

end if

C

C continue until the desired number of test spheres has

C been reached:

C

if (addNp1.lt.deltaN1) go to 101

C

C Write global results:

C

con = pi/6./volume*( (Np1+addNp1)

&	*dint1**3 +(Np+deltaN - Np1-addNp1)

&	*dint2**3 )

C

C Write resulting concentration:

C

write (*,'(a,f10.5)')

&	'final volume fraction = ',con

C

C actual fraction of species 1:

C

if (fracpsd1.ne.1.0) write (*,'(a,f10.7)')

&	'The actual fraction of species 1 is ',

&	float(Np1+addNp1)/float(Np+deltaN)

C

C and finally, number of FIFO spheres:

C

write (*,'(a,i9)') 'number of FIFO spheres = ',deltaN

C

write (*,'(4(a,/))')

&	'OK to proceed?',

&	'yes = 1',

&	'no = other positive integer',

&	'negative to stop'

C

read (*,*) OK

C

if (OK.lt.0) stop

C

if (OK.eq.1) go to 555

C

C Do it all over again (OK ≠ 0, 1, or 2 and OK positive)

C

do i = 1,NN

x(i) = 0.0

y(i) = 0.0

z(i) = 0.0

flagspec(i) = .false.

u(i) = 0.0

v(i) = 0.0

w(i) = 0.0

end do

C

go to 222

C

else

C

go to 222

C

C End of statement:	if (OK.eq.2) then

C

end if

C

C end of statement:	if (con.ge.condes) then

C

end if

C

CCC

C Calculate dimensions of the statistics grid cells:

CCC

C

555	continue

C

write (*,'(a)') 'Statistics grid cells:'

333	write (*,'(a)') 'Enter # of cells in ylen:'

read (*,*) Ngy

C

celly = ylen/Ngy

Ngx = int(xlen/celly)

cellx = xlen/Ngx

errorx = abs(cellx-celly)/(cellx+celly)*2.

Ngz = int(zlen/celly)

cellz = zlen/Ngz

errorz = abs(cellz-celly)/(cellz+celly)*2.

C

C Attempt to match the areas of a grid column in the

C straight section (cellx*ylen) and that of a grid fan in the

C curved section (pi/2Nalpha)*(outrad^2-inrad^2):

C

Nalpha = int(Pi/2.*(outrad+inrad)/cellx)

C

C Begin changes of version 39f.

C

C Make sure that Nalpha is an even number if only a quarter of the

C cell is considered:

C

if (flagquar.and. (2*(Nalpha/2)).ne.Nalpha ) Nalpha =

&						Nalpha + 1

C

C End changes of version 39f.

C

areacell = cellx*ylen

areafan = (Pi/2./Nalpha)*(outrad**2 - inrad**2)

errora = abs(areacell-areafan)/(areacell+areafan)*2.

C

C Write proposed statistics grid cell configuration:

C

write (*,'(a)') 'Proposed configuration:'

write (*,'(3(i3,a),/,i3,a,/,3(a,f6.4,/))')

&	Ngx,' cells along x; ',

&	Ngy,' cells along y; ',

&	Ngz,' cells along z; ',

&	Nalpha,' cells around circular section.',

&	'Relative change between cellx and celly: ',

&	errorx,

&	'between column and fan areas: ',

&	errora,

&	'and between cellz and celly: ',

&	errorz

write (*,'(a,/,a)')

&	'OK to register?  --  yes = 1; no = 0; stop < 0:'

C

read(*,*) OK

C

if (OK.ne.1) then

if (OK.lt.0) stop

go to 333

end if

C

CCCCCCCCCCCCCCCCCCCCC

C Output results:

CCC

C

nul =0.0

time0 = 0.0

C

write (*,'(A,/,a,f15.10)') 'Initial time step ?',

&      'recommended is (dx-d)*0.1/(vbelt-vwall) =  ',

&      (dx-max(dint1,dint2))*0.1/(vbelt-vwall)

Read (*,*) dt

C

C New in v36:

C

write (*,'(a,/)') 'species 1 first'

write (*,'(a,/)') 'normal restitution coefficients:'

C

write (*,*) 'binary collisions of species 1 spheres?'

read (*,*) ed1

C

write (*,*) 'collisions with inner wall bumps?'

read (*,*) eib1

C

write (*,*) 'collisions with outer wall bumps?'

read (*,*) eob1

C

write (*,*) 'collisions with near flat wall?'

read (*,*) enw1

C

write (*,*) 'collisions with far flat wall?'

read (*,*) efw1

C

write (*,'(a,/)') 'friction coefficients:'

C

write (*,*) ' friction of species 1 spheres?'

read (*,*) mu1

C

write (*,*) 'friction with inner wall bumps?'

read (*,*) muib1

C

write (*,*) 'friction with outer wall bumps?'

read (*,*) muob1

C

write (*,*) 'friction with near flat wall?'

read (*,*) munw1

C

write (*,*) 'friction with far flat wall?'

read (*,*) mufw1

C

write (*,'(a,/)') 'tangential restitution coefficients:'

C

write (*,*) 'tangential restitution of species 1 spheres?'

read (*,*) beta01

C

write (*,*) 'tangential restitution of inner wall bumps?'

read (*,*) beta0ib1

C

write (*,*) 'tangential restitution of outer wall bumps?'

read (*,*) beta0ob1

C

write (*,*) 'tangential restitution of near flat wall?'

read (*,*) beta0nw1

C

write (*,*) 'tangential restitution of far flat wall?'

read (*,*) beta0fw1

C

C Species 2:

C

write (*,'(a,/)') 'species 2'

write (*,*) 'same parameters as species 1? - yes = 1'

read (*,*) samepar

C

if (samepar.eq.1) then

C

ed2=ed1

mu2=mu1

beta02=beta01

ed12=ed1

mu12=mu1

beta012=beta01

eib2=eib1

muib2=muib1

beta0ib2=beta0ib1

eob2=eob1

muob2=muob1

beta0ob2=beta0ob1

enw2=enw1

munw2=munw1

beta0nw2=beta0nw1

efw2=efw1

mufw2=mufw1

beta0fw2=beta0fw1

C

else

C

write (*,'(a,/)') 'normal restitution coefficients:'

C

write (*,*) 'binary collisions of species 1 and 2 spheres?'

read (*,*) ed12

C

write (*,*) 'binary collisions of species 2 spheres?'

read (*,*) ed2

C

write (*,*) 'collisions with inner wall bumps?'

read (*,*) eib2

C

write (*,*) 'collisions with outer wall bumps?'

read (*,*) eob2

C

write (*,*) 'collisions with near flat wall?'

read (*,*) enw2

C

write (*,*) 'collisions with far flat wall?'

read (*,*) efw2

C

write (*,'(a,/)') 'friction coefficients:'

C

write (*,*) 'friction of species 1 on species 2 spheres?'

read (*,*) mu12

C

write (*,*) ' friction of species 2 spheres?'

read (*,*) mu2

C

write (*,*) 'friction with inner wall bumps?'

read (*,*) muib2

C

write (*,*) 'friction with outer wall bumps?'

read (*,*) muob2

C

write (*,*) 'friction with near flat wall?'

read (*,*) munw2

C

write (*,*) 'friction with far flat wall?'

read (*,*) mufw2

C

write (*,'(a,/)') 'tangential restitution coefficients:'

C

write (*,*) 'tangential restitution of species 1 and 2 spheres?'

read (*,*) beta012

C

write (*,*) 'tangential restitution of species 2 spheres?'

read (*,*) beta02

C

write (*,*) 'tangential restitution of inner wall bumps?'

read (*,*) beta0ib2

C

write (*,*) 'tangential restitution of outer wall bumps?'

read (*,*) beta0ob2

C

write (*,*) 'tangential restitution of near flat wall?'

read (*,*) beta0nw2

C

write (*,*) 'tangential restitution of far flat wall?'

read (*,*) beta0fw2

C

end if

C

C      write (*,'(A)') 'restitution of interior spheres?'

C      Read (*,*) ed

C      write (*,'(A)') 'restitution of wall spheres and flat walls?'

C      Read (*,*) ew

C      write (*,'(A)') 'friction of interior spheres?'

C      Read (*,*) mu

C      write (*,'(A)') 'friction of wall spheres and flat walls'

C      Read (*,*) muw

C      write (*,'(A)') 'tangential restitution of interior spheres?'

C      Read (*,*) beta0

C      write (*,'(A)') 'tang rest of wall spheres and flat walls?'

C      Read (*,*) beta0w

C

C

C

write (*,'(A)') 'gravity in the x-direction?'

Read (*,*) gravx

write (*,'(A)') 'gravity in the y-direction?'

Read (*,*) gravy

write (*,'(A)') 'gravity in the z-direction?'

Read (*,*) gravz

C

CCC

C General parameters:

C

open(1,file=fname,form='formatted',status='new')

C

C Output texture of the boundary as an integer and calculate

C the number of hemispheres in each region:

C

if (flagb1) then

C

text1 = 1

C

Nw1 = ( (Nwaz1 / 2) + 1)*( (Nwax1+Nwaxf1+1) / 2 ) +

&		( (Nwaz1+1) / 2)*( (Nwax1+Nwaxf1) / 2)

C

else

C

if (flagc1) then

text1 = 0

Nw1 = (Nwax1+Nwaxf1)*Nwaz1

else

text1 = 2

Nw1 = (Nwax1+Nwaxf1)

Nwaz1 = 1

end if

C

end if

C

if (flagb2) then

C

text2 = 1

C

Nw2 = ( (Nwaz2 / 2) + 1)*( (Nwax2+Nwaxf2+1) / 2 ) +

&		( (Nwaz2+1) / 2)*( (Nwax2+Nwaxf2) / 2)

C

else

C

if (flagc2) then

text2 = 0

Nw2 = (Nwax2+Nwaxf2)*Nwaz2

else

text2 = 2

Nw2 = (Nwax2+Nwaxf2)

Nwaz2 = 1

end if

C

end if

C

if (flagb3) then

C

text3 = 1

C

Nw3 = ( (Nwaz3 / 2) + 1)*( (Nwax3+Nwaxf3+1) / 2 ) +

&		( (Nwaz3+1) / 2)*( (Nwax3+Nwaxf3) / 2)

C

else

C

if (flagc3) then

text3 = 0

Nw3 = (Nwax3+Nwaxf3)*Nwaz3

else

text3 = 2

Nw3 = (Nwax3+Nwaxf3)

Nwaz3 = 1

end if

C

end if

C

if (flagb4) then

C

text4 = 1

C

Nw4 = ( (Nwaz4 / 2) + 1)*( (Nwax4+Nwaxf4+1) / 2 ) +

&		( (Nwaz4+1) / 2)*( (Nwax4+Nwaxf4) / 2)

C

else

C

if (flagc4) then

text4 = 0

Nw4 = (Nwax4+Nwaxf4)*Nwaz4

else

text4 = 2

Nw4 = (Nwax4+Nwaxf4)

Nwaz4 = 1

end if

C

end if

C

if (flagbb) then

C

textb = 1

C

Nb = ( (Nbez / 2) + 1)*( (Nbex+1) / 2 ) +

&		( (Nbez+1) / 2)*(Nbex / 2)

C

else

C

if (flagcb) then

textb = 0

if (.not.flagchain) then

Nb = Nbex*Nbez

else

Nb = Nchax*Nlix*Nbez

end if

else

textb = 2

if (.not.flagchain) then

Nb = Nbex

else

Nb = Nchax*Nlix

end if

Nbez = 1

end if

C

end if

C

write (1,'(I9)') Np+deltaN

write (1,'(2(5x,I5),5x,i1,5x,f10.5,1x,i5,1x,f10.5,1x,f10.8)')

&	Nwax1,Nwaz1,text1,waspx10,Nwaxf1,waspxf1,dwal/2.

if (Nwax2.ne.0) then

radius = dwal/2.

else

radius = 0.0

end if

C

C Begin changes of version 39e.

C

C Axisymmetric cells are such that both inner and outer walls are

C uniform; indicate this by making text2 = 5:

C

if (flagaxi) text2 = 5

C

C End changes of version 39e.

C

C

C Begin changes of version 39f.

C

if (flagquar) text2 = 6

C

C End changes of version 39f.

C

write (1,'(2(5x,I5),5x,i1,5x,f10.5,1x,i5,1x,f10.5,1x,f10.8)')

&	Nwax2,Nwaz2,text2,waspx20,Nwaxf2,waspxf2,radius

if (Nwax3.ne.0) then

radius = dwal/2.

else

radius = 0.0

end if

write (1,'(2(5x,I5),5x,i1,5x,f10.5,1x,i5,1x,f10.5,1x,f10.8)')

&	Nwax3,Nwaz3,text3,waspx30,Nwaxf3,waspxf3,radius

if (Nwax4.ne.0) then

radius = dwal/2.

else

radius = 0.0

end if

write (1,'(2(5x,I5),5x,i1,5x,f10.5,1x,i5,1x,f10.5,1x,f10.8)')

&	Nwax4,Nwaz4,text4,waspx40,Nwaxf4,waspxf4,radius

if (flagchain) then

write (1,'(2(10x,I5),10x,i1,1x,f10.8,2x,i5,2x,f10.8)')

&			Nchax,Nbez,textb,dbel/2.,Nlix,hecha

else

write (1,'(2(10x,I5),10x,i1,1x,f10.8,2x,i5,2x,f10.8)')

&			Nbex,Nbez,textb,dbel/2.,Nlix,hecha

end if

C	write (1,'(2(10x,I5),10x,i1,1x,f10.8)') Nbex,Nbez,textb,dbel/2.

write (1,'(4(10x,I5))') Nxlen,Nylen,Nzlen,Ninrad

write (1,'(4F10.5)') cell,dint1 / 2.,dint2 / 2.,dens2

write (1,'(2F15.10)') time0,dt

C

C	write (1,'(6F10.5)') ed,ew,mu,muw,beta0,beta0w

C

C New to v36:

C

write (1,'(f5.4,8(1x,f5.4))')	ed1,mu1,beta01,

&					ed12,mu12,beta012,

&					ed2,mu2,beta02

C

write (1,'(f5.4,5(1x,f5.4))')	eib1,muib1,beta0ib1,

&					eib2,muib2,beta0ib2

C

write (1,'(f5.4,5(1x,f5.4))')	eob1,muob1,beta0ob1,

&					eob2,muob2,beta0ob2

C

write (1,'(f5.4,5(1x,f5.4))')	enw1,munw1,beta0nw1,

&					enw2,munw2,beta0nw2

C

write (1,'(f5.4,5(1x,f5.4))')	efw1,mufw1,beta0fw1,

&					efw2,mufw2,beta0fw2

C

write (1,'(5F12.7)') gravx,gravy,gravz,vbelt,vwall

write (1,'(4(10x,I5))') Ngx,Ngy,Nalpha,Ngz

C

CCC

C Interior spheres:

C

do i = 1,Np

C

write (1,'(3F15.10,5x,L1,5x,i9)') x(i),y(i),z(i),

&					flagspec(i),gerinul

write (1,'(2(e19.13,2x),e19.13)') u(i),v(i),w(i)

write (1,'(2(e19.13,2x),e19.13)') nul,nul,nul

C

end do

C

CCC

C Additional FIFO spheres, if any:

C

if (deltaN.gt.0) then

do i = 1,deltaN

C

C Initialize all velocities and positions to zero for FIFO spheres:

C

write (1,'(3F15.10,5x,L1,5x,i9)') nul,nul,nul,

&				flagspec(i+Np),-i

write (1,'(2(e19.13,2x),e19.13)') nul,nul,nul

write (1,'(2(e19.13,2x),e19.13)') nul,nul,nul

C

end do

end if

C

CCC

C Recommend dimensions of arrays in the main program:

CCC

C Calculate integer dimensions of the cell:

CCC

C

nxa = 1

nxb = nylen + 1

nxc = nylen + ninrad

nxd = nylen + ninrad + nxlen + 1

nxe = nylen + ninrad + nxlen + ninrad

nxf = nylen + ninrad + nxlen + ninrad + nylen

C

nya = 1

nyb = nylen

nyc = nylen + ninrad + ninrad + 1

nyd = nylen + ninrad + ninrad + nylen

C

nza = 1

nzb = nzlen

C

write (*,'(a,/)') 'number of spheres:'

C

if (deltaN.gt.0) then

write (*,'(a,i9)') 'interior + FIFO: ',Np+deltaN

else

write (*,'(a,i9)') 'interior: ',Np

end if

C

write (*,'(a,i6)')

&		'number of arranged species 1 = ',Np1

if (fracpsd1.ne.1.0) write (*,'(a,i6)')

&		'number of arranged species 2 = ',Np - Np1

if (deltaN.ne.0) write (*,'(a,i6)')

&		'number of FIFO spheres = ',deltaN

C

Ntotal = Np + deltaN + Nb +

&			Nw1 + Nw2 + Nw3 + Nw4 + 4

C

write (*,'(2(a,i9))') 'inner wall: ',Nb,

&				' cum: ',Np+deltaN+Nb

write (*,'(2(a,i9),/)') 'outer wall: ',

&	Nw1+Nw2+Nw3+Nw4,' cum: ',

&	Np+deltaN+Nb+Nw1+Nw2+Nw3+Nw4

C

write (*,'(a)') 'recommended array dimensions:'

C

C Begin changes of version 39f.

C

if (flagquar) then

C

write (*,'(a,i9)') 'dimi1 >= ',nxlen

write (*,'(a,i9)') 'dimj1 >= ',nyb

write (*,'(a,i9)') 'dimi2 >= ',nxc

write (*,'(a,i9,/)') 'dimj2 >= ',nyd/2

write (*,'(a,i9)') 'dimk >= ',nzb

write (*,'(a,i9,/)') 'Np >= ',Ntotal

write (*,'(a,i9)') 'Nsx >= ',max(Nalpha/2,Ngy)

write (*,'(a,i9)') 'Nsy >= ',Ngz

C

else

C

write (*,'(a,i9)') 'dimi1 > ',nxlen

write (*,'(a,i9)') 'dimj1 > ',nyb

write (*,'(a,i9)') 'dimi2 > ',nxc

write (*,'(a,i9,/)') 'dimj2 > ',nyd

write (*,'(a,i9)') 'dimk > ',nzb

write (*,'(a,i9,/)') 'Np > ',Ntotal

write (*,'(a,i9)') 'Nsx > ',max(2*(Ngx+Nalpha),Ngy)

write (*,'(a,i9)') 'Nsy > ',max(Ngy,Ngz)

C

end if

C

C Begin changes of version 39f.

C

write (*,'(a,i9)') 'Nchainx > ',Nchax

write (*,'(a,i9,/)') 'NNesc > ',deltaN

C

write (*,'(a,i2)') 'imax = ',imax

write (*,'(a,i2,/)') 'imaxwall = ',imaxwall

C

C Calculate the number of reinsertion points:

C

C

C Begin changes of version 39e.

C

C If the cell is axisymmetric, the following instructions are superfluous,

C since there are only two reinsertion points:

C

if (flagaxi) then

C

C Begin changes of version 39f.

C

if (flagquar) then

nrein = 1

write (*,'(a)') 'Make NREx = NREy = NREz = 1'

write (*,'(a)') 'Make NRExyz = 1'

C

else

C

nrein = 2

write (*,'(a)') 'Make NREx = NREy = NREz = 1'

write (*,'(a)') 'Make NRExyz = 2'

C

end if

C

C End changes of version 39f.

C

else

C

C End changes of version 39e.

C

nx0 = 1

nxfin = nxlen

nxdomain = nxfin - nx0 + 1

nreinx = nxdomain / (2*imax+1)

C

write (*,'(a,i6)') 'Make NREx > ',nreinx

C

ny0 = 1 + max(1+int(dmax/cell),

&				nya+imaxwall)

nyfin = min(nylen - int(dmax/cell),

&				nylen - imaxwall) - 1

nydomain = nyfin - ny0 + 1

nreiny = nydomain / (2*imax+1)

C

write (*,'(a,i6)') 'Make NREy > ',nreiny

C

nz0 = nza + imaxwall + 1

nzfin = nzlen - imaxwall - 1

nzdomain = nzfin - nz0 + 1

nreinz = nzdomain / (2*imax+1)

C

write (*,'(a,i6)') 'Make NREz > ',nreinz

C

nrein = 2*nreinx*nreiny*nreinz

C

write (*,*)

&	'Make NRExyz = 2  *  NREx*NREy*NREz'

C

C Begin changes of version 39e.

C

end if

C

C End changes of version 39e.

C

C

C TEST

C

C For verification purposes, enter total number of reinsertion points,

C number of spheres in the "purgatory", and number of spheres waiting

C above reinsertion points at the time of input:

C

C TEST

C

npurg = deltaN

countwait = 0

C

C TEST

C

write (1,*) nrein,npurg,countwait

C

C TEST

C

Close (1)

C

stop

end

C

CCCC

C

C SU	FUNCTION RAN0(IDUM)

C SU

C SU	implicit none

C SU

C SU	real*4 v(97)

C SU	real*4 dum,ran,ran0,y

C SU	integer*4 idum,iff,iseed,j

C SU

C SU	DATA IFF /0/

C SU	IF(IDUM.LT.0.OR.IFF.EQ.0)THEN

C SU	IFF=1

C SU	ISEED=ABS(IDUM)

C SU	IDUM=1

C SU	DO 11 J=1,97

C SU	DUM=RAN(ISEED)

C SU 11	CONTINUE

C SU	DO 12 J=1,97

C SU	V(J)=RAN(ISEED)

C SU 12	CONTINUE

C SU	Y=RAN(ISEED)

C SU	ENDIF

C SU	J=1+INT(97.*Y)

C SU	IF(J.GT.97.OR.J.LT.1)PAUSE

C SU	Y=V(J)

C SU	RAN0=Y

C SU	V(J)=RAN(ISEED)

C SU	RETURN

C SU	END

C

********************************************************************

C

C GF	FUNCTION RAN1(IDUM)

C GF	implicit none

C GF	real*8 R(97)

C GF	real*8 rm1,rm2,ran1

C GF	integer*4 m1,m2,m3,ia1,ia2,ia3,ic1,ic2,ic3

C GF	integer*4 idum,iff,ix1,ix2,ix3,j

C GF	PARAMETER (M1=259200,IA1=7141,IC1=54773,RM1=3.8580247E-6)

C GF	PARAMETER (M2=134456,IA2=8121,IC2=28411,RM2=7.4373773E-6)

C GF	PARAMETER (M3=243000,IA3=4561,IC3=51349)

C GF	DATA IFF /0/

C GF	IF (IDUM.LT.0.OR.IFF.EQ.0) THEN

C GF	IFF=1

C GF	IX1=MOD(IC1-IDUM,M1)

C GF	IX1=MOD(IA1*IX1+IC1,M1)

C GF	IX2=MOD(IX1,M2)

C GF	IX1=MOD(IA1*IX1+IC1,M1)

C GF	IX3=MOD(IX1,M3)

C GF	DO J=1,97

C GF	IX1=MOD(IA1*IX1+IC1,M1)

C GF	IX2=MOD(IA2*IX2+IC2,M2)

C GF	R(J)=(FLOAT(IX1)+FLOAT(IX2)*RM2)*RM1

C GF	end do

C GF	IDUM=1

C GF	ENDIF

C GF	IX1=MOD(IA1*IX1+IC1,M1)

C GF	IX2=MOD(IA2*IX2+IC2,M2)

C GF	IX3=MOD(IA3*IX3+IC3,M3)

C GF	J=1+(97*IX3)/M3

C GF	IF(J.GT.97.OR.J.LT.1)PAUSE

C GF	RAN1=R(J)

C GF	R(J)=(FLOAT(IX1)+FLOAT(IX2)*RM2)*RM1

C GF	RETURN

C GF	END

C

********************************************************************